// ASYNCHRONOUS FIFO: Handles data transfer between CPU and Memory clock domains

module async_fifo #(parameter DEPTH = 16, ADDR_W = 4) (

input  wire wr_clk, rd_clk, rst, wr_en, rd_en,

input  wire [31:0] din,

output wire [31:0] dout,

output wire full, empty

);

reg [31:0] mem [0:DEPTH-1];

reg [ADDR_W:0] wbin, wgray, rbin, rgray, rq2_wgray, wq2_rgray, rq1_wgray, wq1_rgray;

function [ADDR_W:0] bin2gray(input [ADDR_W:0] b); bin2gray = (b >> 1) ^ b; endfunction

always @(posedge wr_clk or posedge rst) begin

if (rst) begin wbin <= 0; wgray <= 0; end

else if (wr_en && !full) begin mem[wbin[ADDR_W-1:0]] <= din; wbin <= wbin + 1; wgray <= bin2gray(wbin + 1); end

end

always @(posedge rd_clk or posedge rst) begin

if (rst) begin rbin <= 0; rgray <= 0; end

else if (rd_en && !empty) begin rbin <= rbin + 1; rgray <= bin2gray(rbin + 1); end

end

assign dout = mem[rbin[ADDR_W-1:0]];

always @(posedge wr_clk) {rq2_wgray, rq1_wgray} <= {rq1_wgray, rgray};

always @(posedge rd_clk) {wq2_rgray, wq2_rgray} <= {wq1_rgray, wgray};

assign empty = (rgray == wq2_rgray);

assign full  = (wgray == {~wq2_rgray[ADDR_W:ADDR_W-1], wq2_rgray[ADDR_W-2:0]});

endmodule

// DMA CONTROLLER: Manages the data transfer logic and CPU registers

module dma_ctrl (

input wire clk, rst, reg_wr_en,

input wire [4:0] reg_addr,

input wire [31:0] reg_wr_data,

output reg [31:0] reg_rd_data,

output reg [31:0] src_addr, dst_addr, dst_data,

input wire [31:0] src_data,

output reg dst_we, fifo_wr_en, fifo_rd_en,

input wire fifo_full, fifo_empty,

output reg [31:0] fifo_din,

input wire [31:0] fifo_dout,

output reg irq

);

reg [31:0] src_base, dst_base, length, src_ptr, dst_ptr, count;

reg start, busy, done;

reg [2:0] state;

always @(posedge clk or posedge rst) begin

if (rst) begin src_base <= 0; dst_base <= 0; length <= 0; start <= 0; end

else if (reg_wr_en) begin

case (reg_addr)

5'h00: src_base <= reg_wr_data;

5'h04: dst_base <= reg_wr_data;

5'h08: length   <= reg_wr_data;

5'h0C: start    <= reg_wr_data[0];

endcase

end

end

always @(posedge clk or posedge rst) begin

if (rst) begin state <= 0; busy <= 0; done <= 0; irq <= 0; end

else begin

fifo_wr_en <= 0; fifo_rd_en <= 0; dst_we <= 0;

case (state)

0: if (start) begin busy <= 1; src_ptr <= src_base; dst_ptr <= dst_base; count <= length; state <= 1; end

1: begin src_addr <= src_ptr; state <= 2; end

2: if (!fifo_full) begin fifo_din <= src_data; fifo_wr_en <= 1; state <= 3; end

3: if (!fifo_empty) begin fifo_rd_en <= 1; state <= 4; end

4: begin dst_addr <= dst_ptr; dst_data <= fifo_dout; dst_we <= 1; state <= 5; end

5: begin src_ptr <= src_ptr + 4; dst_ptr <= dst_ptr + 4; count <= count - 1; state <= (count == 1) ? 6 : 1; end

6: begin busy <= 0; done <= 1; irq <= 1; state <= 0; end

endcase

end

end

endmodule

// MEMORY MODELS: Source and Destination SRAM

module src_mem (input clk, input [31:0] addr, output reg [31:0] data);

reg [31:0] mem [0:255];

initial begin for (integer i=0; i<256; i=i+1) mem[i] = i + 100; end

always @(posedge clk) data <= mem[addr[9:2]];

endmodule

module dst_mem (input clk, input [31:0] addr, input [31:0] data, input we);

reg [31:0] mem [0:255];

always @(posedge clk) if (we) mem[addr[9:2]] <= data;

endmodule

// TOP LEVEL: The final system-on-chip module

module top_dma_soc (

input wire clk_cpu, clk_mem, rst,

input wire reg_wr_en,

input wire [4:0] reg_addr,

input wire [31:0] reg_wr_data,

output wire [31:0] reg_rd_data,

output wire irq

);

wire [31:0] s_addr, s_data, d_addr, d_data, f_din, f_dout;

wire d_we, f_full, f_empty, f_wr, f_rd;

dma_ctrl dma (

.clk(clk_cpu), .rst(rst), .reg_wr_en(reg_wr_en), .reg_addr(reg_addr),

.reg_wr_data(reg_wr_data), .reg_rd_data(reg_rd_data), .irq(irq),

.src_addr(s_addr), .src_data(s_data), .dst_addr(d_addr), .dst_data(d_data), .dst_we(d_we),

.fifo_full(f_full), .fifo_empty(f_empty), .fifo_wr_en(f_wr), .fifo_rd_en(f_rd),

.fifo_din(f_din), .fifo_dout(f_dout)

);

async_fifo fifo (.wr_clk(clk_cpu), .rd_clk(clk_mem), .rst(rst), .wr_en(f_wr), .rd_en(f_rd), .din(f_din), .dout(f_dout), .full(f_full), .empty(f_empty));

src_mem src (.clk(clk_cpu), .addr(s_addr), .data(s_data));

dst_mem dst (.clk(clk_mem), .addr(d_addr), .data(d_data), .we(d_we));

endmodule